# Initial List of Materials

Oct 22, 2026

Project: Catarman LGU Climate-Resilient SMART Traffic Light (scale prototype) Course: Application Development & Emerging Technologies — Midterm Requirement #2 Prepared by: Squad A — Embedded Systems & Hardware

The prototype uses 1 ESP32 as the main controller, 4 sensor types, LEDs and an OLED as outputs, and a 60 × 60 cm foam-board diorama. All core electronics come from the team's two existing kits — the Super Starter Kit for UNO R3 and the ESP32 Basic Starter Kit — so no new microcontrollers or sensors are bought. Estimated out-of-pocket cost is ₱685–820, within the ₱1,000 team cap; items marked Buy are the only purchases. Prices are approximate local estimates.

## A. Microcontrollers

| # | Item | Qty | Specification | Purpose | Source | Est. cost (₱) |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | ESP32 Development Board (30-pin) | 1 | Dual-core Xtensa LX6, onboard Wi-Fi/Bluetooth, 3.3 V logic, Micro-USB | Main traffic signal controller: runs the signal state machine, reads all sensors, drives all LEDs, talks to the server over MQTT | ESP32 Basic Starter Kit | 0 |
| A2 | Arduino UNO R3 | 1 | ATmega328P, 5 V logic | Benchtop test harness for developing sensor debouncing before moving code to the ESP32 (not wired into the diorama) | Super Starter Kit for UNO R3 | 0 |

## B. Sensors

| # | Item | Qty | Specification | Purpose | Source | Est. cost (₱) |
| --- | --- | --- | --- | --- | --- | --- |
| B1 | HC-SR04 ultrasonic sensor | 1 | 2–400 cm range, 5 V; ECHO needs a 1 kΩ / 2 kΩ voltage divider for the ESP32 | Stop-line vehicle presence on the North approach (N–S phase); backup when the camera is blocked | Super Starter Kit for UNO R3 | 0 |
| B2 | Infrared obstacle avoidance module | 1 | Digital output, 3.3–5 V, adjustable range | Stop-line vehicle presence on the East approach (E–W phase), so both phases have a backup sensor without buying a second HC-SR04 | ESP32 Basic Starter Kit | 0 |
| B3 | Water level detection sensor module | 1 | Analog output, powered from 3.3 V | Curb/gutter flood detection; triggers Flashing Amber mode after 5 s of continuous water | Super Starter Kit for UNO R3 | 0 |
| B4 | Smartphone camera (vision sensor) | 1 | 1080p, wired USB tethering to the laptop | Overhead video feed for vehicle detection and counting (YOLOv8) | Member-owned | 0 |

## C. Actuators & Output Devices

| # | Item | Qty | Specification | Purpose | Source | Est. cost (₱) |
| --- | --- | --- | --- | --- | --- | --- |
| C1 | 5 mm LED — Red | 4 (10 available) | Diffused | Red signal light, one per approach | Both kits | 0 |
| C2 | 5 mm LED — Yellow | 4 (10 available) | Diffused | Yellow signal light / Flashing Amber flood mode | Both kits | 0 |
| C3 | 5 mm LED — Green | 4 (10 available) | Diffused | Green signal light, one per approach | Both kits | 0 |
| C4 | 0.96" OLED display (SSD1306, I²C) | 1 | 128 × 64 px, monochrome | Local status screen: current mode (Level 1–4), phase, countdown timer, power source | ESP32 Basic Starter Kit | 0 |

Opposite approaches share a phase (North = South, East = West), so the 12 LEDs need only 6 ESP32 output pins.

## D. Power Supply & Passive Components

| # | Item | Qty | Specification | Purpose | Source | Est. cost (₱) |
| --- | --- | --- | --- | --- | --- | --- |
| D1 | MB102 breadboard power supply module | 1 | 3.3 V / 5 V rails, DC barrel input, onboard 100 µF capacitors | Steps the 9 V battery down to 5 V for the backup power rail | Super Starter Kit for UNO R3 | 0 |
| D2 | 9 V alkaline battery + 9 V clip with 5.5 mm barrel plug | 3 batteries (1 on hand), 1 clip | PP3 | Backup (auxiliary) power during brownouts; fresh battery for each demo run | 1 battery + clip on hand; Buy 2 batteries | 150 |
| D3 | 1N4007 rectifier diode | 2 | 1 A, ~0.7–1.0 V drop | Diode-OR failover: lets the backup battery take over instantly when main power is cut, without rebooting the ESP32 | Super Starter Kit for UNO R3 | 0 |
| D4 | Resistor 220 Ω | 12 | ¼ W | LED current limiting, one per LED | Both kits | 0 |
| D5 | Resistors 1 kΩ and 2 kΩ | 1 and 1 | ¼ W (2 kΩ = two 1 kΩ in series if none in the kit) | Voltage divider so the HC-SR04's 5 V ECHO signal doesn't damage the 3.3 V ESP32 | Both kits | 0 |
| D6 | Resistor 10 kΩ | 2 | ¼ W | Divider that lets the ESP32 detect whether main power or the backup battery is active | ESP32 Basic Starter Kit | 0 |
| D7 | 830-point breadboard | 2 | — | One for the diorama controller circuit, one for the UNO test harness | One from each kit | 0 |
| D8 | Jumper wires (M-M, F-M, F-F) | 65 + 40 in kits, plus 1 extra set | 20 cm Dupont | Wiring LEDs and sensors from the controller box out to the diorama | Both kits + Buy 1 extra set | 120 |
| D9 | Optional upgrade: 1N5819 Schottky diode ×2 + 1000 µF capacitor | 2 + 1 | ~0.3 V drop; 16 V | Only if the multimeter shows under 4.4 V on battery: smaller voltage drop and steadier switchover | Buy if needed | 35 |

## E. Computing & Connectivity

| # | Item | Qty | Specification | Purpose | Source | Est. cost (₱) |
| --- | --- | --- | --- | --- | --- | --- |
| E1 | Laptop (edge server) | 1 | Windows 10/11 or Linux, 8 GB RAM or more | Runs YOLOv8 vision, MQTT broker, Node.js/Express backend, MySQL Server and the Next.js dashboard | Member-owned | 0 |
| E2 | Micro-USB cable | 1 | Data-capable | ESP32 power + programming | ESP32 Basic Starter Kit | 0 |
| E3 | Phone USB cable | 1 | Data-capable | Smartphone camera to laptop (USB tethering) | Member-owned | 0 |
| E4 | Phone holder / clamp stand | 1 | Gooseneck or tripod with clamp | Holds the smartphone camera above the intersection | Member-owned or Buy | 0–100 |
| E5 | Mobile hotspot / pocket Wi-Fi | 1 | 2.4 GHz | Private local network between laptop and ESP32 (no internet needed) | Member-owned | 0 |
| E6 | HDMI cable | 1 | — | Laptop to venue Smart TV for the dashboard | Member-owned | 0 |
| E7 | Digital multimeter + alligator clip leads | 1 set | — | Measuring the 5 V rail and verifying the power failover | Auxiliary tools on hand | 0 |

## F. Diorama Build Materials

| # | Item | Qty | Specification | Purpose | Source | Est. cost (₱) |
| --- | --- | --- | --- | --- | --- | --- |
| F1 | Foam board | 2 sheets | 5 mm thick; cut to a 60 × 60 cm base | Diorama base and controller box walls | Buy | 150 |
| F2 | Black cartolina | 2 sheets | — | Road surface for the 4-way intersection | Buy | 20 |
| F3 | White paint pen | 1 | — | Lane markings, stop lines, pedestrian crossings | Buy | 40 |
| F4 | Barbecue sticks or thick straws | 1 pack | ~15 cm poles | Signal poles holding the LED heads | Buy | 20 |
| F5 | Hot glue sticks | 1 pack | Glue gun member-owned | Mounting sensors, LEDs and locking jumper wires in place | Buy | 40 |
| F6 | Small clear plastic cup/container | 1 | ~5 cm deep | Gutter "flood well" holding the water sensor; filled with water during the flood demo | Buy | 20 |
| F7 | Toy scale vehicles | ~8 | Tricycles, motorcycles, cars (same scale) | Test objects for camera detection and ultrasonic sensing | Buy | 100 |
| F8 | Double-sided tape | 1 roll | — | Attaching road sheets and labels | Buy | 25 |
| F9 | Shoebox or small plastic box | 1 | — | Controller enclosure for breadboard, ESP32 and power circuit at the corner of the diorama | Member-owned | 0 |

## G. Budget Summary

| Category | Est. cost (₱) |
| --- | --- |
| Electronics to buy (D2 batteries, D8 jumper set) | 270 |
| Diorama materials (F1–F8) | 415 |
| Optional power upgrade (D9) | 0–35 |
| Phone stand, if no member owns one (E4) | 0–100 |
| Total | 685–820 |
| Team cap | 1,000 |

Notes

- All core electronics come from the Super Starter Kit for UNO R3 and the ESP32 Basic Starter Kit; no microcontrollers or sensors are purchased.

- The kits hold only one HC-SR04, so the East approach uses the ESP32 kit's infrared obstacle module as its stop-line sensor.

- Kit parts not used in this prototype: LCD1602, 7-segment displays, servo, stepper motor, relays, DHT11, PIR sensor, sound sensor, buzzers.